General Safety Information

Cleat types and using the pedals

Releasing the cleats from the pedals

WARNING

– To avoid serious injuries:

The method of release varies according to the type of cleats you are using.

(Check the model number and color of your cleats to determine the proper method of release.)

Pay attention to

SPD pedals are designed so that you release them only when you intend to release. They are not designed to release automatically at times such as when falling off the bicycle.

This is a safety measure to prevent you from losing balance and falling off the bicycle as a result of your feet becoming accidentally released from the pedals.

Use only SPD shoes with this product. Other types of shoes may not release from the pedals, or may release unexpectedly.

Use only Shimano cleats (SM-SH51 / SM-SH56) and tighten the mounting bolts securely to the shoes.

Before attempting to ride with these pedals and shoes, make sure

WARNING

Do not use the pedals and cleats in any way other than as described in these Service Instructions.  The cleats are designed to engage and release from the pedals when the cleats and pedals are facing forward.

See below for instructions  on how to install the cleats. FAILURE TO FOLLOW THESE INSTRUCTIONS MAY RESULT IN SERIOUS PERSONAL  INJURY.

Single release mode cleats (SM-SH51) and multiple

Single release mode cleats:

SM-SH51 (black)

These cleats only release when the heel is twisted outward. They will not release if the heel is twisted in any other direction.

You are able to apply upward force to the pedal, since they will not release unless the foot is twisted outward.

Multiple release mode cleats:

SM-SH56 (gold / Optional accessory)

The cleats can be released by twisting in any direction.

Because the cleats can also be released by lifting your heel, they may become accidentally  released when applying upward force on the pedals.

These cleats should not be used for riding style which may involve

pulling up on the pedals or for activities, such as jumping, in which pulling forces are applied to the pedals causing possible release from the pedals.

the mark

R: right pedal

L : left pedal

15 mm spanner

PD-M520

Tightening torque:

35 N·m {304 in. lbs.} min.

you understand the operation of the engagement / release

release mode cleats (SM-SH56) are available for use with

mechanism for the pedals and cleats (shoes).

Before you attempt to ride with these pedals and shoes, apply the brakes, then place one foot on the ground and practice engaging and releasing each shoe from its pedal until you can do so naturally and with minimal effort.

Ride on level ground first until you become adept at engaging and releasing your shoes from the pedals.

Before riding, adjust the spring tension of the pedals to your liking.

When riding at low speed or when there is a possibility that you might need to stop riding, (for example, when doing a U-turn, nearing an intersection, riding uphill or turning a blind curve), release your shoes from the pedal beforehand so that you can quickly put your feet onto the ground at any time.

Use a lighter spring tension for attaching the pedal cleats when riding in adverse conditions.

Keep cleats and bindings clear of dirt and debris to ensure engagement and release.

If you do not maintain both your shoes and cleats in good

these pedals.

Each type of cleat has its own distinctive features, so be sure to read the Service Instructions  carefully and select whichever type of cleat best suits your riding style and the terrain and riding conditions which you will encounter.

Other types of cleats such as single release mode cleats (SM-SH52) and multiple release mode cleats (SM-SH55) cannot be used, as they do not provide stable enough step-out performance  or sufficient holding force.

Engaging the cleats with the pedals

Press the cleats into the pedals with a forward and downward  motion.

The cleats will not necessarily  release if you lose your balance. Accordingly, for places and conditions where it looks as though you may lose balance, make sure that you have sufficient time to release the cleats beforehand.

When you are pedaling vigorously,  your heel may inadvertently twist outward and this may cause the cleat to release accidentally.  If the cleat releases by accident, you may fall off the bicycle and serious injury may result.

You can help to prevent this by adjusting the amount of force required to release the cleats, and you should practice until you are accustomed  to the amount of force and the angle required to release the cleats.

Single release mode

Release the cleats from the pedals by twisting your heels outward.

Note:

It is necessary  to practice releasing until you become accustomed  to

the technique.

Although the cleats can be released by twisting your heels in any direction or by lifting, they will not necessarily release if you lose your balance.

Accordingly,  for places and conditions  where it looks as though you may lose balance, make sure that you have sufficient time to release the cleats beforehand.

If the cleats release by accident, you may fall off the bicycle and serious injury may result.

If the features of multiple release mode cleats are not sufficiently understood  and if they are not used correctly, then they may release accidentally with a greater frequency than for single release mode cleats.

You can help to prevent this by adjusting the amount of force required to release the cleats in any direction, and you should practice until you are accustomed to the amount of force and the angle required to release.

Multiple release mode

Release the cleats from the pedals by twisting your heels in any direction.

Note:

In multiple release mode, it is necessary  to practice releasing until you become accustomed  to

Adjusting the spring tension of the binding

The spring force is adjusted by means of adjustment bolts. The adjustment bolts are located behind each of the bindings, and there are two

adjustment  bolts on each pedal. Equalize the tensions by referring to the adjustment plates and by counting the number of turns of the adjustment bolts. The spring tension can be adjusted in four steps for each turn of the adjustment bolt.

Adjustment bolt

Adjustment plate

2

1

Adjustment bolt

Decrease 	Increase

condition, release and engagement to the pedals could become

unpredictable or difficult, which could result in severe injury.

Remember to check the cleats periodically for wear. When the cleats are worn, replace them.

the technique. Releasing by lifting	2	1

your heel requires particular practice.

Always check the spring tension after replacing the pedal cleats and before riding.

Be sure to attach reflectors to the bicycle when riding at night. Do not continue riding the bicycle if the reflectors are dirty or damaged, otherwise it becomes more difficult for oncoming vehicles to see you.

Obtain and read the service instructions carefully prior to

PD-M970

SI-41R0F

From the bottom of the shoe, position a cleat and then a cleat adapter over the cleat holes. The cleats are compatible with both left and right pedals. Provisionally tighten the cleat mounting bolts.

3.

Provisional tightening torque

Waterproof seal

Remove the sockliner and attach the waterproof  seal.

PD-M970: 2.5 mm Allen key

PD-M540 / PD-M520: 3 mm Allen key

When the adjustment bolt is turned clockwise, the spring tension increases, and when it is turned counterclockwise, the spring tension decreases.

installing the parts. Loose, worn, or damaged parts may cause

PD-M540

SPD Pedals

for cleat mounting bolts:

Note:

Waterproof seal

Strongest position 	Weakest position

serious injury to the rider.

We strongly recommend only using genuine Shimano replacement parts.

PD-M520

2.5 N·m {22 in. lbs.}	The waterproof seal is supplied with Shimano shoes which require this step to be

Adjustment bolt

If you have any questions concerning your pedals, contact a professional dealer.

Read these Technical Service Instructions carefully, and keep them in a safe place for later reference.

BE SURE TO READ AND FOLLOW THE ABOVE WARNINGS CAREFULLY.

Technical Service Instructions

Attaching the cleats

With a pair of pliers or a similar tool, pull off the rubber cover to expose the cleat mounting holes.

1.

4 mm Allen key

Cleat mounting bolts

Cleat adapter

Cleat

carried out.

Sockliner

Adjustment plate

If the adjustment  plate is at the strongest or the weakest position, do not turn the adjustment  bolt any further.

If the warnings are not followed, your shoes may not come out of the pedals when you intend or they may come out unexpectedly or accidentally, and severe injury may result.

Note:

This step may not be necessary  depending

Front	Note:

SM-SH51

In order to prevent accidental  release from occurring,  make sure

all the spring tensions are properly adjusted.

NOTE:

Check that there is no looseness in any joints or connections before riding the bicycle. (BB-FC, FC-PD)

In addition, if pedaling performance  does not feel normal, check this once more.

An optional reflector set (SM-PD22) is available. Please contact a professional  dealer for details.

Parts are not guaranteed  against natural wear or deterioration resulting from normal use.

For maximum performance  we highly recommend  Shimano lubricants and maintenance  products.

on the type of shoes.

Rubber cover for cleat mounting holes

SPD shoe

2. Remove the sockliner and position a cleat nut over the oval holes.

Cleat nut

Note:

This step may not be necessary depending on the type of shoes.

Sockliner

Position the triangular portion

of the cleat toward the front of the shoe.

Adjusting the cleat position

The cleat has an adjustment  range of 20 mm front to back and 5 mm right to left. After provisionally  tightening the cleat, practice engaging and releasing, one shoe at

1.

a time. Readjust to determine  the best cleat position.

After you have determined  the best cleat position, firmly tighten the cleat mounting bolts with a 4 mm Allen key.

2.

Tightening torque:

5 – 6 N·m {43 – 52 in. lbs.}

Mounting the pedals on the crank arms

Use an 8 mm Allen key (for PD-M970 / PD-M540 pedals) or a 15 mm spanner (for PD-M520 pedals) to mount the pedals on the crank arms. The right pedal has a right-hand  thread; the left pedal has a left-hand thread.

PD-M970

PD-M540

8 mm Allen key

Tightening torque:

35 N·m {304 in. lbs.} min.

The spring tensions should be adjusted equally on both sides of each pedal and for both right and left pedals. If they are not adjusted equally, it can cause the rider difficulty to engage or release from the pedals.

Cleat replacement

Cleats wear out over time and should be replaced periodically.  Cleats should be replaced when it becomes difficult to release, or it starts to release with much less effort than the when it was in new condition.

Maintenance of the axle units

If you experience any trouble with the rotating parts of the pedal, the pedal may require adjustment.  Obtain advice from a professional  dealer.

One Holland, Irvine, California 92618, U.S.A. Phone: +1-949-951-5003	Industrieweg  24, 8071 CT Nunspeet, The Netherlands  Phone: +31-341-272222

3-77 Oimatsu-cho,  Sakai-ku, Sakai, Osaka 590-8577, Japan

Please note: Specifications  are subject to change for improvement  without notice. (English)

C  Mar. 2006 by Shimano Inc. PIT. IZM. Printed in Malaysia